🌿

GREENHOUSE IoT SYSTEM

Complete Project Documentation

ESP32  ·  MQTT  ·  Node.js  ·  TimescaleDB  ·  Docker  ·  GitLab CI/CD

Version 1.0  ·  3 June 2026

# 1. Executive Summary

The Greenhouse IoT System is a full-stack, production-grade automation platform designed to monitor and control a physical greenhouse. It connects embedded hardware (ESP32 microcontrollers) to a cloud-hosted backend through an MQTT message broker, stores time-series sensor data in a purpose-built time-series database, and exposes a secure REST API consumed by both a web dashboard and a native Android application.

The system autonomously manages temperature regulation (windows and fans), irrigation scheduling, and soil moisture monitoring — with fallback logic embedded in the firmware itself so the greenhouse continues to operate safely even if the internet connection is lost.

Alerts are delivered via email and SMS when any sensor reading crosses a configurable threshold. All infrastructure runs in Docker containers orchestrated by Docker Compose, and code deployments are fully automated through a GitLab CI/CD pipeline.

| Metric | Value |
| --- | --- |
| Hardware platform | ESP32 (dual-core 240 MHz, built-in WiFi) |
| Communication protocol | MQTT over TCP/IP (QoS 1, LWT support) |
| Backend runtime | Node.js 20 LTS + Express 4 |
| Database | TimescaleDB (PostgreSQL 16 extension) |
| Pre-computed aggregates | 5-minute, 1-hour, 1-day continuous aggregates |
| Data retention | Automatic: compress after 7 days, drop after 1 year |
| Authentication | JWT Access (15 min) + Refresh tokens (30 days) |
| Containerisation | Docker Compose — 6 services |
| CI/CD | GitLab Pipelines: test → build → SSH deploy |
| Notifications | Email (SMTP/Nodemailer) + SMS (Twilio / SMSAPI / Vonage) |
| SSL | Let's Encrypt via Certbot — auto-renewed every 12 hours |
| Irrigation schedules | Unlimited cron-based programs (per-device, per-day) |

# 2. System Architecture

## 2.1 High-Level Architecture

The system is divided into three physical tiers that communicate over a local network and the internet:

- Edge Tier — ESP32 firmware reads sensors and switches relays. It publishes JSON payloads over MQTT and subscribes to command topics. Autonomous threshold logic keeps the greenhouse safe when offline.

- Server Tier — A Linux server running Docker hosts: the MQTT broker (Mosquitto), the Node.js REST API, TimescaleDB, Redis, and Nginx as the SSL-terminating reverse proxy.

- Client Tier — A React web dashboard and a React Native Android app consume the REST API over HTTPS with JWT-authenticated requests.

## 2.2 Communication Flow

Every 30 seconds the ESP32 publishes a JSON payload to the topic greenhouse/SERA_001/sensors. The backend MQTT service receives it, writes a row into the sensor_readings hypertable, and passes the values to the alert engine. The alert engine evaluates all active rules; if any threshold is violated and the per-rule cooldown has elapsed it inserts a record into alert_history and dispatches email and/or SMS notifications.

The frontend dashboard polls GET /api/sensors/history with an interval parameter (5min, 1h, 1d). The backend routes that query to the matching TimescaleDB continuous aggregate view, returning pre-computed averages in sub-millisecond time regardless of how many raw rows exist.

Commands (open windows, start pump, etc.) flow in the reverse direction: the web or mobile app calls POST /api/actuators/command; the backend publishes a JSON message to greenhouse/SERA_001/cmd/windows (or /cmd/pump, /cmd/fan, /cmd/auto); the ESP32 receives it, executes the action, and immediately publishes a heartbeat with the updated state.

## 2.3 Technology Stack Rationale

| Layer | Technology | Why This Choice |
| --- | --- | --- |
| Microcontroller | ESP32 (PlatformIO / Arduino) | Built-in WiFi + Bluetooth, dual-core, rich ecosystem, 12-bit ADC for analog soil sensor, 5V-tolerant relay drivers |
| Message Bus | Mosquitto MQTT 2 | Lightweight publish-subscribe ideal for IoT; LWT detects device offline; QoS 1 guarantees at-least-once delivery |
| Backend | Node.js + Express | Non-blocking I/O handles many concurrent MQTT messages and API requests efficiently; vast npm ecosystem |
| Time-series DB | TimescaleDB | PostgreSQL syntax + automatic hypertable partitioning + continuous aggregates = 10–100× faster range queries vs plain SQL; built-in compression saves 90 % disk space; retention policies automatic |
| Cache / Queue | Redis | Sub-millisecond session storage; used for rate-limit state and irrigation job queue |
| Reverse Proxy | Nginx | SSL termination, static file serving, WebSocket upgrade for future MQTT-WS dashboard |
| SSL | Let's Encrypt + Certbot | Free, automated certificate renewal every 90 days |
| CI/CD | GitLab CI/CD | Native to GitLab repos; multi-stage pipeline with Docker registry; SSH deploy with zero-downtime rolling restart |

# 3. Complete Folder & File Structure

The repository is organised so that each implementation stage (Etapa 1 through 10) stands alone and can be deployed independently. The final production layout merges the backend, docker, and scripts directories.

greenhouse-iot/                          ← Repository root

│

├── firmware/                            ← All ESP32 source code

│   ├── etapa1/                          ← Stage 1: sensors + relays only (no WiFi)

│   │   ├── platformio.ini               ← PlatformIO board & library config

│   │   └── src/

│   │       ├── config.h                 ← ALL constants: pins, thresholds, timing

│   │       ├── main.cpp                 ← setup() + loop() entry point

│   │       ├── sensors.h / .cpp         ← DHT22 + capacitive soil sensor

│   │       └── actuators.h / .cpp       ← Relay control + autonomous threshold logic

│   │

│   └── etapa2/                          ← Stage 2: adds WiFi + MQTT + NTP

│       ├── platformio.ini               ← Adds PubSubClient + ArduinoJson libs

│       └── src/

│           ├── config.h                 ← Extended: WiFi creds, MQTT host, topics

│           ├── main.cpp                 ← Extended loop: publishes every 30s, heartbeat

│           ├── sensors.h / .cpp         ← Same as Stage 1 + JSON serialisation

│           ├── actuators.h / .cpp       ← Same + pump timer check

│           └── wifi_mqtt.h / .cpp       ← WiFi connect, MQTT pub/sub, NTP, LWT

│

├── backend/                             ← Node.js server

│   ├── package.json                     ← Dependencies: express, pg, mqtt, nodemailer…

│   ├── Dockerfile                       ← Multi-stage build: deps → production image

│   ├── db/

│   │   └── init/

│   │       └── 001_schema.sql           ← Full TimescaleDB schema, aggregates, seed data

│   └── src/

│       ├── app.js                       ← Express app, middleware stack, route mounting

│       ├── db/

│       │   └── pool.js                  ← pg Pool wrapper: query(), queryOne(), transaction()

│       ├── mqtt/

│       │   └── service.js               ← MQTT client: receives sensors, dispatches commands

│       ├── scheduler/

│       │   └── irrigation.js            ← node-cron: loads DB schedules, fires pump commands

│       ├── alerts/

│       │   └── engine.js                ← Evaluates alert rules, calls email/SMS, logs history

│       ├── notifications/

│       │   ├── email.js                 ← Nodemailer HTML email builder

│       │   └── sms.js                   ← Multi-provider SMS (Twilio / SMSAPI / Vonage)

│       ├── utils/

│       │   └── logger.js                ← Winston: console + rotating file logs

│       └── api/

│           ├── middleware/

│           │   └── auth.js              ← JWT verify middleware + adminOnly guard

│           └── routes/

│               ├── auth.js              ← POST /login, /refresh, /logout, GET /me

│               ├── sensors.js           ← GET /latest, /history, /stats, /export

│               ├── actuators.js         ← POST /command, GET /events

│               ├── schedules.js         ← CRUD irrigation schedules + manual trigger

│               ├── alerts.js            ← CRUD alert rules + history + acknowledge

│               ├── devices.js           ← GET devices + online status

│               └── dashboard.js         ← Aggregated dashboard endpoint

│

├── docker/

│   ├── mosquitto/

│   │   └── config/

│   │       └── mosquitto.conf           ← Auth, persistence, WebSocket listener

│   └── nginx/

│       └── default.conf                 ← HTTP→HTTPS redirect + SSL + proxy to backend

│

├── docker-compose.yml                   ← 6-service stack: tsdb, mqtt, redis, backend, nginx, certbot

├── .env.example                         ← Template for all secrets (never commit .env!)

│

├── scripts/                             ← One-command automation

│   ├── install_server.sh                ← Full Ubuntu setup: Docker, UFW, systemd service

│   ├── start.sh                         ← docker compose up -d --build

│   ├── stop.sh                          ← docker compose down

│   ├── backup.sh                        ← pg_dump → gzip, auto-deletes >30-day backups

│   ├── ssl.sh                           ← Certbot initial certificate issue

│   ├── security_harden.sh               ← SSH hardening, fail2ban, Docker daemon security

│   ├── health_check.sh                  ← Checks all containers, API, MQTT, disk, RAM

│   ├── db_shell.sh                      ← Opens psql inside the TimescaleDB container

│   ├── mqtt_add_user.sh                 ← Adds authenticated MQTT user + restarts broker

│   └── logs.sh                          ← docker compose logs -f [service]

│

├── .gitlab-ci.yml                       ← CI/CD: test → build Docker image → SSH deploy

└── tests/

└── e2e.test.js                      ← Jest end-to-end: auth, sensors, alerts, security

# 4. Database Design — TimescaleDB

## 4.1 Why TimescaleDB Over MySQL

TimescaleDB is a PostgreSQL extension that adds automatic time-partitioning (hypertables), pre-computed continuous aggregate views, transparent compression, and data retention policies. For a greenhouse sensor system that writes one row every 30 seconds per device, this translates to approximately 2,880 rows/day. After one year that is over one million rows in sensor_readings alone — exactly the scenario where TimescaleDB shines.

| Feature | TimescaleDB | MySQL |
| --- | --- | --- |
| Time-range queries | 10–100× faster via chunk pruning | Full table scan or manual partitioning |
| time_bucket() grouping | Native SQL function | Not available; requires complex FLOOR() math |
| Pre-computed aggregates | Continuous Aggregates (auto-refresh) | Requires manual stored procedures |
| Compression | Automatic after N days; up to 95 % saving | None built-in |
| Retention policy | One SQL call; runs automatically | Cron job + DELETE query |
| SQL compatibility | Full PostgreSQL | MySQL dialect only |

## 4.2 Tables Overview

| Table | Type | Purpose |
| --- | --- | --- |
| sensor_readings | Hypertable (partitioned by day) | Raw sensor data: temperature, humidity, soil moisture per device per timestamp |
| sensor_5min | Continuous Aggregate (5-min buckets) | Pre-averaged data for dashboard charts covering up to 48 hours |
| sensor_1h | Continuous Aggregate (1-hour buckets) | Pre-averaged data for 7-day trend charts |
| sensor_1d | Continuous Aggregate (1-day buckets) | Pre-averaged data for 30-day and annual reports |
| actuator_events | Hypertable | Every relay action: who triggered it, when, duration |
| irrigation_schedules | Regular table | Cron-based watering programs: name, expression, duration, enabled flag |
| irrigation_log | Hypertable | Execution history of every irrigation cycle |
| alert_rules | Regular table | Configurable threshold rules: parameter, operator, value, cooldown, notification flags |
| alert_history | Hypertable | Every alert fired, whether email/SMS was sent, acknowledgement status |
| users | Regular table | Application users: bcrypt passwords, role (admin/viewer), notification prefs |
| refresh_tokens | Regular table | SHA-256 hashed JWT refresh tokens with expiry and revocation flag |
| settings | Regular table | Key-value application configuration store |

# 5. Firmware — ESP32 (C++ / Arduino)

## 5.1 Stage 1 — Autonomous Operation

The Stage 1 firmware operates completely independently — no WiFi, no server required. This is intentional: the greenhouse must be safe even if the internet goes down or the server restarts. The firmware reads sensors every 30 seconds, applies threshold logic, and controls all five relays accordingly.

### Hardware connected to ESP32

| GPIO | Component | Direction | Notes |
| --- | --- | --- | --- |
| 4 | DHT22 DATA | INPUT | 10 kΩ pull-up resistor to 3.3 V required |
| 34 | Soil sensor AO | INPUT (ADC) | GPIO 34 is ADC-only; cannot be used as OUTPUT |
| 16 | Relay — Window 1 | OUTPUT | Active LOW: relay energises when GPIO goes LOW |
| 17 | Relay — Window 2 | OUTPUT | Second window motor; same timing as Window 1 |
| 18 | Relay — Pump/Valve | OUTPUT | 220 V solenoid valve; safety: normally-closed position |
| 19 | Relay — Fan 1 | OUTPUT | 220 V ventilation fan |
| 21 | Relay — Fan 2 | OUTPUT | 220 V ventilation fan |
| 2 | Status LED | OUTPUT | Built-in LED; ON while pump is running |

### Relay debounce and motor travel time

Each relay has a software debounce of 2 seconds (RELAY_DEBOUNCE_MS) to prevent rapid switching that could damage motor windings. Window relays are held on for WINDOW_TRAVEL_TIME_MS (15 seconds by default) — the time needed for the linear actuator or geared motor to fully open or close — then released automatically. This means the motor is never left energised while stationary.

### Configurable thresholds (config.h)

| Constant | Default | Behaviour |
| --- | --- | --- |
| TEMP_OPEN_WINDOWS | 28 °C | Both window relays fire if temperature exceeds this value |
| TEMP_CLOSE_WINDOWS | 24 °C | Windows close when temperature drops to this value (hysteresis gap) |
| TEMP_START_FANS | 26 °C | Both fans start above this temperature |
| TEMP_STOP_FANS | 23 °C | Fans stop below this temperature |
| TEMP_ALERT_HIGH | 35 °C | Critical alert; opens windows and fans unconditionally |
| TEMP_ALERT_LOW | 5 °C | Critical alert; closes windows and stops fans unconditionally |
| SOIL_ADC_DRY | 3200 | ADC reading in open air — maps to 0 % moisture |
| SOIL_ADC_WET | 800 | ADC reading submerged in water — maps to 100 % moisture |
| SOIL_ALERT_DRY_PCT | 20 % | Alert published when computed soil moisture is below this |

## 5.2 Stage 2 — WiFi, MQTT, and NTP

Stage 2 extends Stage 1 with three major additions:

- WiFi with retry logic: the firmware attempts to connect at startup (up to WIFI_TIMEOUT_S seconds) and retries every WIFI_RETRY_MS milliseconds in the main loop. The greenhouse threshold logic continues running whether or not WiFi is available.

- MQTT with Last Will Testament (LWT): when the broker connection is established, a retained LWT message {"online":false} is registered. If the ESP32 drops off the network, the broker automatically publishes this message so the server can detect the device going offline without waiting for a heartbeat timeout.

- NTP time synchronisation: on first WiFi connection, the firmware calls configTzTime() with the Romanian timezone string. All subsequent MQTT payloads include a Unix timestamp ts field so the backend can store accurate wall-clock times even if the server and device clocks drift.

### MQTT topic design

| Topic | Direction | Retained | Purpose |
| --- | --- | --- | --- |
| greenhouse/SERA_001/sensors | ESP32 → Server | Yes | Full sensor reading + relay state, published every 30 s |
| greenhouse/SERA_001/heartbeat | ESP32 → Server | Yes | Lightweight ping every 60 s; also acts as LWT target |
| greenhouse/SERA_001/alerts | ESP32 → Server | No | Immediate critical alerts (e.g. TEMP_ALERT_HIGH exceeded) |
| greenhouse/SERA_001/cmd/windows | Server → ESP32 | No | Payload: {"action":"OPEN"} or {"action":"CLOSE"} |
| greenhouse/SERA_001/cmd/pump | Server → ESP32 | No | Payload: {"action":"ON","duration_ms":60000} or {"action":"OFF"} |
| greenhouse/SERA_001/cmd/fan | Server → ESP32 | No | Payload: {"action":"ON","fan":1} (fan 0 = all fans) |
| greenhouse/SERA_001/cmd/auto | Server → ESP32 | No | Payload: {"action":"ON"} enables autonomous threshold mode |

# 6. Backend — Node.js API Server

## 6.1 Application Structure

The backend follows a layered architecture. The Express application (app.js) wires together middleware and mounts route handlers. Each route handler calls database queries through pool.js or dispatches commands through mqtt/service.js. Cross-cutting concerns (authentication, logging, rate limiting) are handled as Express middleware before the route handlers execute.

## 6.2 API Endpoint Reference

| Method | Endpoint | Auth | Description |
| --- | --- | --- | --- |
| POST | /api/auth/login | Public | Returns access_token (15 min) + refresh_token (30 days) |
| POST | /api/auth/refresh | Refresh token | Issues new token pair; old refresh token is revoked (rotation) |
| POST | /api/auth/logout | Bearer | Revokes the supplied refresh token |
| GET | /api/auth/me | Bearer | Returns authenticated user profile |
| GET | /api/sensors/latest | Bearer | Most recent reading per device |
| GET | /api/sensors/history | Bearer | Time-bucketed history; auto-selects the right aggregate view |
| GET | /api/sensors/stats | Bearer | Min/max/avg for a period (24h, 7d, 30d) |
| GET | /api/sensors/export | Bearer | Download raw data as CSV or JSON |
| POST | /api/actuators/command | Bearer | Send a relay command to the device via MQTT |
| GET | /api/actuators/events | Bearer | Paginated log of all relay actions |
| GET | /api/schedules | Bearer | List all irrigation schedules |
| POST | /api/schedules | Admin | Create a new cron-based irrigation program |
| PUT | /api/schedules/:id | Admin | Update schedule (name, cron, duration, enabled) |
| DELETE | /api/schedules/:id | Admin | Disable schedule (soft-delete) |
| POST | /api/schedules/trigger | Bearer | Manually run pump for N seconds right now |
| GET | /api/schedules/log | Bearer | History of executed irrigation cycles |
| GET | /api/alerts/rules | Bearer | List all threshold alert rules |
| POST | /api/alerts/rules | Admin | Create a new alert rule |
| PUT | /api/alerts/rules/:id | Admin | Modify threshold, notifications, cooldown |
| GET | /api/alerts/history | Bearer | Paginated alert history with optional acknowledged filter |
| PUT | /api/alerts/history/:id/ack | Bearer | Acknowledge an alert (marks as reviewed) |
| GET | /api/devices | Bearer | List devices with computed online status |
| GET | /api/dashboard | Bearer | Combined: latest reading, 24h stats, recent events, active alerts |
| GET | /health | Public | Health check — DB and MQTT connectivity |

## 6.3 Sensor History Query Routing

One of the most important performance decisions is how GET /api/sensors/history routes its query. A naive implementation would always query the raw sensor_readings table, resulting in slow aggregation over millions of rows as the system ages. The backend instead maps the requested interval to the most appropriate pre-computed view:

| interval parameter | Data source | Query cost | Typical use |
| --- | --- | --- | --- |
| 1min | sensor_readings (raw) | Moderate (recent data only) | Last-hour detail view in dashboard |
| 5min | sensor_5min (aggregate) | Near zero | Last 24 hours chart |
| 15min / 30min | sensor_5min (aggregate) | Near zero | Last 48 hours chart |
| 1h / 6h | sensor_1h (aggregate) | Near zero | Last 7 days chart |
| 1d / 7d | sensor_1d (aggregate) | Near zero | Monthly / annual chart |

## 6.4 Authentication & Security

Authentication uses a dual-token JWT pattern:

- Access tokens are short-lived (15 minutes). They are signed with JWT_SECRET and carry the user id, username, and role. They are not stored in the database — verification is purely cryptographic.

- Refresh tokens are long-lived (30 days). They are hashed with SHA-256 and stored in the refresh_tokens table. On each use the old token is revoked and a new one is issued (token rotation). This limits the damage of a stolen refresh token to a single use.

- Password hashing uses bcrypt with cost factor 12.

- Rate limiting is applied globally (200 requests/15 minutes per IP) with a stricter limit on POST /api/auth/login (10 attempts/15 minutes) to prevent brute-force attacks.

- Helmet.js sets security headers (X-Frame-Options, X-Content-Type-Options, Strict-Transport-Security).

# 7. Docker Infrastructure

## 7.1 Services

| Service | Image | Exposed | Role |
| --- | --- | --- | --- |
| timescaledb | timescale/timescaledb:latest-pg16 | localhost:5432 | Primary database; data persisted in tsdb_data volume |
| mosquitto | eclipse-mosquitto:2 | 1883 (MQTT), localhost:9001 (WS) | MQTT broker; password-authenticated; data + log volumes |
| redis | redis:7-alpine | Internal only | Session rate-limit state; LRU eviction at 256 MB |
| backend | Built from ./backend/Dockerfile | Internal only (via Nginx) | Node.js API; depends_on health-check of timescaledb |
| nginx | nginx:1.25-alpine | 80, 443 | SSL termination; proxies /api/* and / to backend |
| certbot | certbot/certbot | Internal only | Webroot challenge; renews certificates every 12 hours |

## 7.2 Data Volumes

- tsdb_data — PostgreSQL data directory. Never delete this volume.

- mosquitto_data — MQTT persistent session and retained message store.

- redis_data — Redis append-only log for durability across restarts.

- certbot_web — Shared webroot between Nginx and Certbot for ACME challenges.

- certbot_certs — Let's Encrypt certificates mounted read-only into Nginx.

## 7.3 Production Deployment Checklist

- Edit .env — fill in all secrets (POSTGRES_PASSWORD, JWT_SECRET, SMTP credentials, SMS API key, DOMAIN).

- Run sudo ./scripts/install_server.sh — installs Docker, configures UFW firewall, creates systemd service.

- Point your domain DNS A record to the server IP. Wait for propagation.

- Run ./scripts/start.sh to start all containers.

- Run ./scripts/ssl.sh to obtain the initial Let's Encrypt certificate.

- Run ./scripts/security_harden.sh to apply SSH hardening, fail2ban, and Docker security options.

- Set up GitLab CI/CD variables (see docs/GITLAB_VARIABLES.md) and push to main to verify the pipeline.

- Change the default admin password immediately: POST /api/auth/change-password.

# 8. GitLab CI/CD Pipeline

Every push to the main branch triggers a three-stage pipeline. The pipeline runs inside GitLab-hosted runners and SSH-deploys to the production server without any manual intervention.

| Stage | Job | What it does | Blocks deploy on failure? |
| --- | --- | --- | --- |
| test | test:backend | npm ci → eslint → jest with coverage report | Yes |
| test | test:firmware | pio run (compile-only, no upload) | No (allow_failure) |
| build | build:docker | docker build → push to GitLab Container Registry | Yes |
| deploy | deploy:production | rsync files → docker compose up -d --build backend | Yes |
| deploy | notify:success | POST to Slack webhook with commit info | No (allow_failure) |

The deployment step uses rsync to copy changed files, explicitly excluding .env, logs, and node_modules. It then runs docker compose up -d --build backend which rebuilds only the backend image and restarts only that container — other services (database, broker, Redis) are not interrupted.

# 9. Alert Engine

The alert engine runs in-process within the backend. It is called after every sensor reading is saved to the database. The engine fetches all enabled alert rules for the device, evaluates each rule's condition against the current reading, and fires notifications when thresholds are crossed — subject to a per-rule cooldown period.

## 9.1 Rule Conditions

Each rule specifies a parameter (temperature, humidity, or soil_pct), an operator (>, <, >=, <=, =), and a numeric threshold. Examples of built-in default rules:

| Rule name | Condition | Severity | Default cooldown |
| --- | --- | --- | --- |
| Temperature critical high | temperature > 35 | critical | 30 min |
| Temperature critical low | temperature < 5 | critical | 30 min |
| Humidity too high | humidity > 90 | warning | 30 min |
| Soil too dry | soil_pct < 20 | warning | 30 min |

## 9.2 Notification Delivery

When a rule fires: (1) a record is inserted into alert_history; (2) the engine fetches all users who have notify_email or notify_sms enabled; (3) for each qualifying user it calls the email or SMS notifier asynchronously; (4) it updates the alert_history record with email_sent and sms_sent flags; (5) the in-memory cooldown map is updated to prevent duplicate alerts during the cooldown window.

# 10. Irrigation Scheduler

The irrigation scheduler uses node-cron to execute watering cycles at defined times. Schedules are stored in the irrigation_schedules table and can be created, modified, and enabled/disabled through the REST API without restarting the server.

Every five minutes a background cron job reloads active schedules from the database. New schedules are registered; schedules that were disabled or deleted have their cron jobs stopped. This design allows real-time UI changes to take effect within five minutes.

When a schedule fires: the scheduler publishes a MQTT command to start the pump with the configured duration; records a row in irrigation_log with started_at; waits asynchronously for the duration; then updates irrigation_log with ended_at, duration_actual_s, and success=true. On any error, success=false and the error message are recorded.

Example cron expressions for irrigation:

| Cron expression | Meaning |
| --- | --- |
| 0 7 * * 1,3,5 | Every Monday, Wednesday, Friday at 07:00 |
| 0 7,19 * * * | Every day at 07:00 and 19:00 |
| 0 6 * * 1-5 | Weekdays only at 06:00 |
| */30 * * * * | Every 30 minutes (testing only) |

# 11. Automation Scripts Reference

| Script | Run as | Description |
| --- | --- | --- |
| scripts/install_server.sh | sudo | Full Ubuntu/Debian setup: Docker CE, Docker Compose v2, UFW firewall rules, fail2ban, systemd greenhouse.service, auto-generates JWT secrets, creates MQTT password file |
| scripts/start.sh | user | Pulls latest images, docker compose up -d --build, prints service status |
| scripts/stop.sh | user | docker compose down (volumes preserved) |
| scripts/backup.sh | user / cron | pg_dump ¦ gzip → /opt/greenhouse/backups/sera_db_YYYYMMDD_HHMMSS.sql.gz. Deletes backups older than 30 days |
| scripts/ssl.sh | user | Runs certbot certonly --webroot for the configured DOMAIN. Run once after DNS is set up |
| scripts/security_harden.sh | sudo | Disables root SSH, forces key-only auth, configures fail2ban jails, sets Docker daemon security options, registers daily backup cron |
| scripts/health_check.sh | user | Checks each Docker container status, API /health, MQTT port 1883, TimescaleDB pg_isready, disk usage, and RAM usage. Exits non-zero on any failure |
| scripts/db_shell.sh | user | Opens an interactive psql session inside the timescaledb container |
| scripts/mqtt_add_user.sh | user | Usage: ./mqtt_add_user.sh <username> <password>. Appends to Mosquitto password file and restarts broker |
| scripts/logs.sh | user | Usage: ./logs.sh [service]. Tails docker compose logs for the named service (default: backend) |
| firmware/etapa1/scripts/setup.sh | sudo | Installs PlatformIO CLI, configures udev rules for ESP32 USB, adds user to dialout group |
| firmware/etapa1/scripts/flash.sh | user | Usage: ./flash.sh [upload¦monitor¦all¦clean]. Compiles and uploads firmware |
| firmware/etapa2/scripts/set_wifi.sh | user | Interactive prompt: SSID, password, MQTT host/user/pass → patches config.h automatically |
| firmware/etapa1/scripts/calibrate_soil.sh | user | Guides user through dry/wet ADC readings and patches SOIL_ADC_DRY / SOIL_ADC_WET in config.h |
| tests/scripts/run_tests.sh | user | npm install + API_URL=<url> jest — runs full E2E suite against a live server |

# 12. Quick-Start Setup Guide

## 12.1 Firmware (ESP32)

- Install VS Code + PlatformIO IDE extension.

- Open the firmware/etapa2/ folder in VS Code.

- Run: chmod +x scripts/*.sh && sudo ./scripts/setup.sh

- Run: ./scripts/set_wifi.sh — enter your WiFi SSID, password, and server IP.

- Run: ./scripts/calibrate_soil.sh — calibrate the soil sensor.

- Run: ./scripts/flash.sh all — compiles, uploads, and opens Serial Monitor.

- Type STATUS in Serial Monitor to verify all sensors are reading correctly.

## 12.2 Server

- Copy the repository to your Linux server (Ubuntu 22.04+ recommended).

- Run: sudo ./scripts/install_server.sh — installs all dependencies.

- Edit .env with your database password, JWT secrets, SMTP credentials, and domain.

- Point your domain DNS A record to the server public IP.

- Run: ./scripts/start.sh — all six Docker containers start.

- Run: ./scripts/ssl.sh — issues the Let's Encrypt certificate.

- Run: ./scripts/security_harden.sh — applies production hardening.

- Navigate to https://yourdomain.com — log in with admin / Admin1234! and change the password immediately.

## 12.3 GitLab CI/CD

- Push the repository to GitLab.

- In GitLab: Settings → CI/CD → Variables — add all variables listed in docs/GITLAB_VARIABLES.md.

- Push a commit to main — the pipeline runs automatically.

- Monitor: CI/CD → Pipelines.

# 13. Security Considerations

- Never commit .env to version control. Add it to .gitignore immediately.

- Change the default admin password (Admin1234!) as the very first action after deployment.

- MQTT port 1883 is unencrypted. Restrict it at the firewall to the local network only (UFW rule: ufw allow from 192.168.0.0/16 to any port 1883). For internet-facing MQTT, use MQTTS (8883) with TLS.

- JWT_SECRET and JWT_REFRESH_SECRET should each be at least 64 random bytes. The install script generates these automatically with openssl rand -hex 64.

- The backup script stores unencrypted database dumps. If backups are stored offsite, encrypt them first with gpg --symmetric.

- Run ./scripts/health_check.sh from a cron job every 5 minutes and alert yourself if it exits non-zero.

Greenhouse IoT System  ·  Complete Project Documentation  ·  v1.0